CYPRUS INTERNATIONAL UNIVERSITY

MICROPROCESSORS

CMPE324 / EELE324

GROUP MEMBERS:

1. AUGUSTIN KABWE KALIMBA 22326035

2. DEFI MUKEBO KASONGO        22323612

3.  MELIK KOÇHAN                        22207971

Directed by:

Asst. Prof. Dr. FUAT UYGUROĞLU

Asst. Prof. Dr. Öykü AKAYDIN

FINAL TERM PROJECT

Department of Electrical-Electronic and computer Engineering

Nicosia, Turkish Republic of Northern Cyprus

2025/2026

- Introduction

This final report presents the complete design and implementation of a Hexadecimal Countdown Display system using an Arduino Uno microcontroller and a single-digit 7-segment display. The system continuously counts down from F (15) to 0 in hexadecimal, displaying each digit for one second before cycling back.

The project demonstrates practical skills in embedded systems design, including GPIO control, software delay implementation, and lookup-table-based segment encoding. The ATMega328P microcontroller on the Arduino Uno platform was selected for its widespread support and robust digital I/O capabilities.

- System Overview & Block Diagram

The system is organized into three functional modules:

- Power Supply Module: The Arduino Uno is powered via USB (5V). This voltage is used directly to drive the digital output pins.

- Microcontroller Module (ATMega328P): Handles all logic — iterating through the countdown sequence, looking up segment bitmasks, and writing output signals to GPIO pins.

- Display Module: A common-anode 7-segment display connected to digital pins 2–9 through 220 Ω current-limiting resistors.

- Final Circuit Diagram & Hardware Implementation

The circuit was implemented on a breadboard. Each segment of the 7-segment display (a through g) is connected to an Arduino digital pin through a 220 Ω resistor. The decimal point (DP) pin is held permanently LOW (off). The common-anode pin of the display is connected to the Arduino's 5V supply.

| Segment | Arduino Pin | Variable Name |
| --- | --- | --- |
| a | Pin 7 | Aseg |
| b | Pin 6 | Bseg |
| c | Pin 4 | Cseg |
| d | Pin 3 | Dseg |
| e | Pin 2 | Eseg |
| f | Pin 8 | Fseg |
| g | Pin 9 | Gseg |
| DP (decimal point) | Pin 5 | DPseg (always LOW) |

- Final pictures of the circuit

The following photographs show the assembled hardware during operation:

Figure 1 – Display showing the digit 'F' (start of countdown)

Figure 2 – Display showing digit '7' mid-countdown

Figure 3 – Arduino Uno board with multi-color wiring to the breadboard

- Complete Program Code

// Segment pins

const int Eseg = 2;   // E Segment on the 7-Segment Display is connected to DIGITAL Pin 2

const int Dseg = 3;   // D Segment on the 7-Segment Display is connected to DIGITAL Pin 3

const int Cseg = 4;   // C Segment on the 7-Segment Display is connected to DIGITAL Pin 4

const int DPseg = 5;  // DP (Decimal Point) Segment on the 7-Segment Display is connected to DIGITAL Pin 5

const int Bseg = 6;   // B Segment on the 7-Segment Display is connected to DIGITAL Pin 6

const int Aseg = 7;   // A Segment on the 7-Segment Display is connected to DIGITAL Pin 7

const int Fseg = 8;   // F Segment on the 7-Segment Display is connected to DIGITAL Pin 8

const int Gseg = 9;   // G Segment on the 7-Segment Display is connected to DIGITAL Pin 9

// {A,B,C,D,E,F,G}

// Storing which segments will light up for a number or hexadecimal character in memory.

// Holds the pin states corresponding to hexadecimal numbers in A-G order.

byte hexDigits[16][7] = {

{1,1,1,1,1,1,0}, // 0

{0,1,1,0,0,0,0}, // 1

{1,1,0,1,1,0,1}, // 2

{1,1,1,1,0,0,1}, // 3

{0,1,1,0,0,1,1}, // 4

{1,0,1,1,0,1,1}, // 5

{1,0,1,1,1,1,1}, // 6

{1,1,1,0,0,0,0}, // 7

{1,1,1,1,1,1,1}, // 8

{1,1,1,1,0,1,1}, // 9

{1,1,1,0,1,1,1}, // A

{0,0,1,1,1,1,1}, // b

{1,0,0,1,1,1,0}, // C

{0,1,1,1,1,0,1}, // d

{1,0,0,1,1,1,1}, // E

{1,0,0,0,1,1,1}  // F

};

// The purpose of this code is to configure the 7-segment display pins as outputs and turn off the decimal point.

// Function used to send/display whether the output from the pin to the segment is HIGH or LOW.

void setup() {

pinMode(Aseg, OUTPUT);

pinMode(Bseg, OUTPUT);

pinMode(Cseg, OUTPUT);

pinMode(Dseg, OUTPUT);

pinMode(Eseg, OUTPUT);

pinMode(Fseg, OUTPUT);

pinMode(Gseg, OUTPUT);

pinMode(DPseg, OUTPUT);

digitalWrite(DPseg, LOW); // Decimal point is turned off

}

// Sends the state to each segment and controls whether the segment is LOW(0) or HIGH(1).

void displayDigit(int num) {

digitalWrite(Aseg, !hexDigits[num][0]); // Sends 0 if 1, and 1 if 0 (Inverts the logic)

digitalWrite(Bseg, !hexDigits[num][1]);

digitalWrite(Cseg, !hexDigits[num][2]);

digitalWrite(Dseg, !hexDigits[num][3]);

digitalWrite(Eseg, !hexDigits[num][4]);

digitalWrite(Fseg, !hexDigits[num][5]);

digitalWrite(Gseg, !hexDigits[num][6]);

}

void loop() {

// F -> 0 countdown

for(int i = 15; i >= 0; i--) {

displayDigit(i);

delay(1000);}

- }Problems and solutions

Common-Anode Polarity

Challenge: Digits displayed incorrectly because the segments worked in reverse logic.
Solution: Since the display was common-anode, LOW signals were used to turn segments ON. Applying logical NOT (!) fixed the display output.

Segment Mapping

Challenge: Some digits appeared incorrect due to wrong segment connections.
Solution: Each pin was tested individually to identify the correct segment mapping, and the lookup table was updated.

Wiring Issues

Challenge: Incorrect wiring caused flickering and wrong patterns.
Solution: Color-coded wires and systematic checking of each connection improved accuracy and stability.

- CONCLUSION

This project successfully achieved all required objectives. A hexadecimal countdown display system was designed and implemented on the Arduino Uno platform, counting from F (15) down to 0 with a 1-second delay per digit, and looping continuously.

Key learning outcomes include: understanding common-anode vs. common-cathode display logic and polarity inversion, designing and using segment lookup tables for efficient multi-digit encoding, GPIO configuration and digital output control on the ATMega328P, and the trade-offs between software delay and timer-based timing methods.

The project also reinforced good engineering practices: systematic debugging (individual pin probing), clear code structure (named constants, helper functions), and hardware safety (current-limiting resistors). The final system is clean, stable, and fully meets the project specifications.